/*

Ping))) Sensor

This sketch reads a PING))) ultrasonic

rangefinder and returns the distance to the

closest object in range. To do this, it sends a

pulse to the sensor to initiate a reading, then

listens for a pulse to return.  The length of

the returning pulse is proportional to the

distance of the object from the sensor.

The circuit:

* +V connection of the PING))) attached to +5V

* GND connection attached to ground

* SIG connection attached to digital pin 7

http://www.arduino.cc/en/Tutorial/Ping

This example code is in the public domain.

*/

int inches = 0;

int cm = 0;

long readUltrasonicDistance(int triggerPin, int echoPin)

{

pinMode(triggerPin, OUTPUT);  // Clear the trigger

digitalWrite(triggerPin, LOW);

delayMicroseconds(2);

// Sets the trigger pin to HIGH state for 10 microseconds

digitalWrite(triggerPin, HIGH);

delayMicroseconds(10);

digitalWrite(triggerPin, LOW);

pinMode(echoPin, INPUT);

// Reads the echo pin, and returns the sound wave travel time in microseconds

return pulseIn(echoPin, HIGH);

}

void setup()

{

Serial.begin(9600);

}

void loop()

{

// measure the ping time in cm

cm = 0.01723 * readUltrasonicDistance(7, 7);

// convert to inches by dividing by 2.54

inches = (cm / 2.54);

Serial.print(inches);

Serial.print("in, ");

Serial.print(cm);

Serial.println("cm");

delay(100); // Wait for 100 millisecond(s)

}